The research lead analyzed formal specification languages like Estelle, LOTOS, SDL, ASN.1, and Protocol Buffers, and our approach directly builds on that work. As lightweight and practical frameworks for specifying new protocols, the result suggested ASN.1 and Protocol Buffers. This foundation has been used to expand the suggested header fields into an actual packet structure with message flows, fixed field sizes, and defined semantics. This guarantees that the design is robust for Scapy implementation and Wireshark analysis, yet not too complicated to understand in instructional laboratories.

Packet Header Layout

With an optional checksum for error detection, the protocol employs a variable-length payload after a small 8-byte header.

| Field | Size | Type | Description |
| --- | --- | --- | --- |
| Version | 1 B | Uint8 | Identifying the protocol version |
| Type | 1 B | Uint8 | Defining the message ( 0=Request, 1=Ack, 2=Response, 3=Error ) |
| Seq | 2 B | Uint16 | Sequence number for retransmission and sorting. |
| Length | 2 B | Uint16 | Length of payload in bytes |
| CRC16 | 2 B | Uint16 | Optional 16-bit checksum across header and payload. |

Rationale:

- 1 byte each is sufficient because Version and Type are small values.

- 2 bytes (0-65,535) are standard since length and sequence can increase.

- Conventionally, CRC16 is two bytes, widely accepted, and easy to validate.

Packet Layout Diagram

Figure1: custom protocol packet layout

Message Types

- Request (0) - client sent a request to transmit a command or data.

- Ack (1) – confirms that a request was received and echoes the Sequence value.

- Response (2) – an optional data confirmation response from the server.

- Error (3) – this indicates a sequence, CRC mismatch, or a corrupted transmission.

Protocol Behavior

It is easy to observe retransmissions in Wireshark since the protocol uses an easy stop-and-wait ARQ flow.

Custom Protocol State Machine

Figure2: Client-server state machines showing request/ack cycle with retransmission

Use –Case Scenarios

Chat Example

- Clients sent Request (“Hello”, Seq=1).

- Server replied with Ack(1) and optional Response(“Hi !”).

- If not Ack arrives, the client retransmits.

IoT Sensor Example

- Sensor sent Request(“temp=23.5”, Seq=n) in every 5 seconds.

- Gateway replied with Ack(n).

- If a packet is corrupted, the gateway responds with ERROR.

Message Flow Diagrams

Normal Flow

Error Flow

Figures 3 and 4: seq diagrams illustrating normal and error message flows